PING

Portable Intelligent Noise Grid

Project Requirements & Design Specification

Version 0.1  —  July 2026

Author

Joshua Jackson

Acoustics & Signal Processing Engineer

NASA Ames Research Center Intern (2025)

jsj830@outlook.com

linkedin.com/in/joshua-jackson-40a46225a

PING (Portable Intelligent Noise Grid) is a modular 8-channel microphone array research platform for acoustic source localization, beamforming algorithm comparison, and array geometry investigation, initialized at startup via CLAP (Channel Level Acquisition Protocol), a built-in 8-channel synchronization and sensitivity verification test.

# SECTION 1 — PROJECT IDENTITY

## 1.1 Project Name

PING — Portable Intelligent Noise Grid

## 1.2 Document Title

Project Requirements and Design Specification (PRDS)

## 1.3 Version

v0.1 — Initial Release

## 1.4 Date

July 2026

## 1.5 Author

Joshua Jackson

Acoustics & Signal Processing Engineer

NASA Ames Research Center Intern (2025)

## 1.6 One-Sentence Summary

PING (Portable Intelligent Noise Grid) is a modular 8-channel microphone array research platform for acoustic source localization, beamforming algorithm comparison, and array geometry investigation, initialized at startup via CLAP (Channel Level Acquisition Protocol), a built-in 8-channel synchronization and sensitivity verification test.

## 1.7 Sub-Protocols

- PING — Portable Intelligent Noise Grid: the complete hardware and software research platform

- CLAP — Channel Level Acquisition Protocol: the startup self-test sequence that verifies all 8 channels before any measurement begins

# SECTION 2 — MOTIVATION AND BACKGROUND

During acoustic health monitoring research conducted at NASA Ames Research Center in 2025, the author developed a multi-microphone testbed for fault detection in brushless DC motors used on electric UAV systems. The testbed employed four microphones arranged at 0°, 30°, 60°, and 90° elevation angles relative to a motor’s rotor plane, capturing acoustic emissions at different spatial positions for FFT-based spectral analysis and fault classification via convolutional neural networks.

While the system showed promise for identifying distinguishable acoustic signatures between healthy and degraded motor conditions, a fundamental limitation emerged: each microphone was treated as an independent measurement point. Time-of-arrival differences between channels were not exploited, and no spatial localization capability existed. On a multi-rotor platform, where four or more motors operate simultaneously, this creates an inherent ambiguity: spectral anomalies could be detected, but their spatial origin could not be determined. Identifying which specific motor was faulting required physical isolation of each motor, a process made difficult with real-time pre/post-flight monitoring.

This limitation motivated a central research question:

Could a single microphone array, positioned externally to a UAV, localize acoustic fault signatures to a specific motor without requiring per-motor instrumentation?

Answering this question requires capabilities that were absent from the NASA Ames testbed; specifically, synchronized multichannel acquisition, time-delay estimation between microphone pairs, and spatial beamforming algorithms capable of resolving multiple simultaneous acoustic sources.

PING (Portable Intelligent Noise Grid) was conceived as a research platform to develop, validate, and systematically compare these capabilities. The platform is designed not as a single-purpose tool, but as a modular, open system that can be applied across multiple acoustic research scenarios, from laboratory beamforming experiments to field deployment on UAV test platforms.

The long-term vision for PING is contribution back to the aerospace acoustic research community, specifically, exploring whether an external array-based monitoring approach could complement or replace per-motor instrumentation in UAV health monitoring systems.

# SECTION 3 — PROJECT OBJECTIVES

## 3.1 Primary Objectives

The following primary objectives define the minimum success criteria for PING v1.0:

OBJ-P1: Multichannel Acoustic Acquisition

Demonstrate synchronized 8-channel audio acquisition with sample-accurate timing across all microphone positions using the TASCAM US-16x08 interface and Behringer ECM8000 measurement microphones.

OBJ-P2: Sensitivity Calibration

Develop and validate a calibration procedure (CLAP — Channel Level Acquisition Protocol) that equalizes sensitivity across all 8 channels to within ±1 dB, ensuring measurement-grade consistency.

OBJ-P3: Direction of Arrival Estimation

Implement and validate at least three DOA estimation algorithms (GCC-PHAT, Delay and Sum, MUSIC) capable of localizing a single acoustic source to within ±5° accuracy at 1 meter distance in the frequency range 200 Hz – 2,638 Hz.

OBJ-P4: Real-Time Heatmap Visualization

Generate a real-time polar heatmap display of estimated source direction with less than 500ms latency, suitable for live demonstration and field deployment.

OBJ-P5: Array Geometry Implementation

Design, fabricate, and validate all five interchangeable array geometries (Circular, Linear, Rectangular, Spiral, Random) and three interchangeable aperture rings (5.0 cm, 8.5 cm, 15.0 cm) using 3D printed components designed in Onshape.

OBJ-P6: Experimental Documentation

Conduct and document a minimum of seven structured experiments (EXP-001 through EXP-007) following a consistent methodology framework, with publication-quality figures and written reports for each.

## 3.2 Secondary Objectives

The following secondary objectives extend PING beyond baseline functionality:

OBJ-S1: Noise Suppression

Implement adaptive beamforming (MVDR) for directional noise suppression, demonstrating measurable SNR improvement when a target source is isolated from a competing noise source at a different angle.

OBJ-S2: Multi-Source Localization

Extend DOA estimation to simultaneously localize two independent acoustic sources, directly relevant to the multi-motor UAV fault isolation problem identified in Section 2.

OBJ-S3: Geometry Comparison Study

Conduct a controlled experimental comparison of all five array geometries under identical conditions, producing a quantitative performance matrix suitable for inclusion in a technical report or conference submission.

OBJ-S4: Acoustic Camera

Overlay real-time beamforming heatmap onto a synchronized video feed of the measurement environment, creating a visual acoustic camera demonstration.

OBJ-S5: UAV Motor Fault Localization

Apply PING to a brushless DC motor testbed replicating the NASA Ames experimental setup, demonstrating whether an external array can localize fault signatures to a specific motor without per-motor instrumentation.

OBJ-S6: Open Source Release

Publish PING as a fully documented open source repository suitable for use by other acoustic researchers, with a structured README, technical report, and reproducible experimental framework.

OBJ-S7: Acoustic Scene Classification

Integrate a machine learning classification pipeline that processes beamformed audio streams and identifies sound event categories with confidence scores. Classification results displayed as text overlay on the real-time acoustic camera feed. Example output: “Source at 47° — Motor Bearing Fault (71% confidence)”

OBJ-S8: Distributed Acoustic Sensor Network (DASN)

Design a multi-unit PING network protocol enabling three or more synchronized PING units to perform collaborative triangulation of acoustic events across extended outdoor environments. Target applications include wildlife monitoring, perimeter security, and UAV detection. Long-term vision: a deployable three-unit PING network capable of localizing impulsive acoustic events (gunshots, UAV motors, wildlife calls) to within 5 meters at distances up to 500 meters.

# SECTION 4 — FUNCTIONAL REQUIREMENTS

The word SHALL denotes a mandatory requirement. SHOULD denotes a desired but non-mandatory requirement. MAY denotes an optional enhancement.

## 4.1 Hardware Requirements

| ID | Requirement | Metric | Verification | Traced To |
| --- | --- | --- | --- | --- |
| FR-001 | 8-channel simultaneous acquisition | Timing offset < 1 sample (< 22.7 μs) | EXP-001 | OBJ-P1 |
| FR-002 | 44,100 Hz minimum sample rate | Device query confirmation | EXP-001 | OBJ-P1 |
| FR-003 | 24-bit recording depth | ≥96 dB dynamic range | EXP-001 | OBJ-P1 |
| FR-004 | Sensitivity matching ±1 dB post-CLAP | Post-cal RMS deviation < ±1 dB at 1 kHz | EXP-002 | OBJ-P2 |
| FR-005 | 48V phantom power all channels | 44–52V on all 8 channels | Hardware | OBJ-P1 |
| FR-006 | Geometry swap < 10 minutes | Timed physical swap | Physical | OBJ-P5 |
| FR-007 | Ring swap < 2 minutes, no tools | Timed physical swap | Physical | OBJ-P5 |
| FR-008 | 5/8"-27 stand compatibility | Secure with no play | Physical | OBJ-P5 |
| FR-009 | Cable clearance > 5cm from capsules | Ruler measurement | Physical | OBJ-P1,P5 |
| FR-010 | Standardized collar on all 5 mounts | Physical fit test | Physical | OBJ-P5 |

## 4.2 Software Requirements

| ID | Requirement | Metric | Verification | Traced To |
| --- | --- | --- | --- | --- |
| FR-011 | System latency < 500ms | Impulse to display < 500ms | EXP-001 | OBJ-P4 |
| FR-012 | Geometry positions as (2×N) array | Unit test: test_geometry.py | pytest | OBJ-P5 |
| FR-013 | Calibration persists across sessions | File loads in < 1s at startup | EXP-002 | OBJ-P2 |
| FR-014 | GCC-PHAT TDOA error < 2 samples | Known TDOA test signal | EXP-003 | OBJ-P3 |
| FR-015 | DAS accuracy < ±5° | Known angle measurement | EXP-003 | OBJ-P3 |
| FR-016 | MUSIC accuracy < ±5° | Known angle measurement | EXP-003 | OBJ-P3 |
| FR-017 | Heatmap ≤1° resolution, ≥2 Hz refresh | Code review + visual verify | EXP-003 | OBJ-P4 |
| FR-018 | Data logging with metadata | File inspection post-experiment | All EXPs | OBJ-P6 |
| FR-019 | CLAP < 10s, PASS/FAIL per channel | Timed execution | EXP-002 | OBJ-P2 |
| FR-020 | Random array reproducible (seed=42) | Hash comparison across sessions | pytest | OBJ-P5 |

## 4.3 Documentation Requirements

| ID | Requirement | Metric | Verification | Traced To |
| --- | --- | --- | --- | --- |
| FR-021 | All 7 report sections per experiment | Section checklist | All EXPs | OBJ-P6 |
| FR-022 | Figures ≥150 DPI, labeled | File metadata + visual | All EXPs | OBJ-P6 |
| FR-023 | CAD STEP files in repository | Repository file check | GitHub | OBJ-P5,S6 |
| FR-024 | Theory doc per algorithm | Document review | docs/ | OBJ-P6 |
| FR-025 | README enables cold start | Independent user test | GitHub | OBJ-S6 |
| FR-026 | Technical report complete | All sections present | Final | OBJ-P6 |

## 4.4 Secondary Requirements (SHOULD)

| ID | Requirement | Metric | Traced To |
| --- | --- | --- | --- |
| FR-027 | Noise suppression ≥6 dB SNR improvement | ≥6 dB at ≥45° angular separation | OBJ-S1 |
| FR-028 | Multi-source localization ±10° | ≥45° angular separation, 1m | OBJ-S2 |
| FR-029 | Acoustic camera overlay | <100ms temporal alignment | OBJ-S4 |
| FR-030 | Classification accuracy ≥70% | Confusion matrix on test set | OBJ-S7 |
| FR-031 | Distributed network <1ms timing offset | Field test, known source | OBJ-S8 |

# SECTION 5 — SYSTEM ARCHITECTURE

## 5.1 Four-Layer System Overview

PING is organized into four architectural layers. Each layer communicates only with the layer directly above or below it, a principle called separation of concerns that ensures changes in one layer never require changes in another.

- Layer 1 — Hardware: Microphones, Array Mounts, DAQ Interface

- Layer 2 — Signal Processing: Acquisition, Calibration, Preprocessing

- Layer 3 — Algorithms: GCC-PHAT, DAS, MVDR, MUSIC, SRP-PHAT

- Layer 4 — Application: Experiments, Visualization, Reports

## 5.2 Hardware Architecture

The complete hardware signal chain flows from acoustic event through the array assembly to the TASCAM US-16x08 interface and into the host computer:

- Acoustic event produces pressure wave

- ECM8000 capsule converts pressure to analog voltage (microvolts range)

- ECM8000 internal preamp amplifies to balanced analog signal

- 3ft XLR cable (balanced, differential) carries signal with common-mode noise rejection

- TASCAM US-16x08 supplies 48V phantom power, applies mic preamp gain, converts via 24-bit ADC at 44,100 Hz

- USB 2.0 stream delivers digital PCM audio — 8 channels simultaneously — to host computer

- sounddevice library receives the stream into the PING software stack

## 5.3 Array Assembly

The array assembly consists of three standardized interchangeable components:

Microphone Module (fixed across all geometries)

Behringer ECM8000 in a standardized collar module. The collar design is identical across all five array mounts; microphones transfer between geometries without any per-microphone reconfiguration.

Interchangeable Rings (circular array only)

| Ring | Radius | Aliasing Frequency | Best Application |
| --- | --- | --- | --- |
| RING-A | 5.0 cm | 4,476 Hz | High frequency, small sources |
| RING-B | 8.5 cm | 2,638 Hz | Speech, primary use (PETG blue) |
| RING-C | 15.0 cm | 1,494 Hz | Low frequency, UAV motors (PETG green) |

Interchangeable Mounts

| Mount | Geometry | Primary Application |
| --- | --- | --- |
| MOUNT-1 | Circular | 360° localization, demonstrations, algorithm benchmarking |
| MOUNT-2 | Linear | Engine bay scans, exhaust measurements, Car diagnostic |
| MOUNT-3 | Rectangular | Acoustic imaging, near-field measurements, 2D localization |
| MOUNT-4 | Spiral | Grating lobe reduction research, comparison with circular |
| MOUNT-5 | Random | Side lobe behavior comparison with uniform circular array |

## 5.4 Software Data Flow

Audio flows through the software stack in the following sequence:

- Startup: load configuration from ping_config.yaml

- Execute CLAP Protocol — verify all 8 channels, load or compute calibration gains

- Layer 2: oaap.hardware.interface records multichannel audio buffer

- Layer 2: oaap.calibration.sensitivity applies gain correction

- Layer 2: oaap.signal_processing.preprocessing applies DC removal, bandpass filter, Hann window

- Layer 3: selected beamforming algorithm computes spatial spectrum — (360,) array, one value per degree

- Layer 4: oaap.visualization.heatmap renders polar heatmap, marks estimated source direction

- Layer 4: oaap.logging saves raw audio, metadata, and figures

## 5.5 Performance Budget

Total system latency must not exceed 500ms (FR-011). Estimated per-step timing:

| Processing Step | Estimated Time |
| --- | --- |
| Audio capture (1024 samples @ 44,100 Hz) | ~23ms |
| Calibration application | ~1ms |
| Preprocessing (DC, filter, window) | ~5ms |
| MUSIC algorithm (most expensive) | ~50ms |
| Heatmap rendering | ~30ms |
| Display update | ~10ms |
| Total estimated | ~119ms — well within 500ms budget |

# SECTION 6 — HARDWARE DESIGN SPECIFICATION

## 6.1 Microphone Selection — Behringer ECM8000

Requirements satisfied: FR-001, FR-004, FR-005

| Parameter | Value |
| --- | --- |
| Transducer type | Electret condenser |
| Polar pattern | Omnidirectional |
| Frequency response | 15 Hz – 20 kHz |
| Sensitivity | -60 dBV/Pa |
| Self noise | 29 dB-A SPL |
| Max SPL | 130 dB |
| Phantom power | 15 – 48V |
| Output impedance | 600Ω |
| Body diameter | 34mm |

Three primary factors drove microphone selection:

- Omnidirectional pattern — critical for beamforming: directional steering comes from array geometry and signal processing, not individual mic directionality. Cardioid mics introduce direction-dependent coloration that corrupts spatial spectrum calculation.

- Flat frequency response — ECM8000 is marketed as a measurement microphone with intentionally flat response. Accurate DOA estimation requires all channels to respond identically across frequency.

- Unit-to-unit consistency — eight matched units exhibit lower sensitivity variation than mixed models. Post-calibration residual variation is lower across matched units.

Alternatives considered and rejected:

| Alternative | Price (8 units) | Self Noise | Rejection Reason |
| --- | --- | --- | --- |
| Primo EM272Z1 (DIY XLR) | ~$150 | 14 dB-A | Requires custom electronics, higher build complexity, assembly inconsistency risk |
| DPA 4060 matched pair | ~$3,200 | 23 dB-A | Cost prohibitive ($400/mic) |
| Rode NT5 matched pair | ~$800 | 16 dB-A | Cardioid pattern unsuitable for beamforming |

## 6.2 Audio Interface — TASCAM US-16x08

Requirements satisfied: FR-001, FR-002, FR-003, FR-005

The TASCAM US-16x08 was already owned by the author, eliminating acquisition cost. It meets every technical requirement: 8 simultaneous mic preamp inputs (FR-001), 44,100 Hz / 24-bit capability (FR-002, FR-003), per-channel-group 48V phantom power (FR-005), and native sounddevice Python library support via USB 2.0.

## 6.3 Array Mount Design

Manufacturing: FDM 3D printing on Bambu Lab P1S. Final material: PETG. Prototype material: PLA. CAD platform: Onshape (cloud-based, version controlled).

PETG was selected over PLA because: (1) temperature resistance to ~80°C vs PLA’s ~60°C failure point; (2) significantly higher impact resistance for field deployment; (3) better moisture resistance for outdoor applications.

## 6.4 Interchangeable Ring Specification

Aliasing frequency derivation for each ring:

- Adjacent mic spacing: d = 2 × r × sin(π/N) for N mics on radius r

- Aliasing frequency: f_alias = c / (2 × d), where c = 343 m/s

- RING-A (5cm): d = 3.83cm → f_alias = 4,476 Hz

- RING-B (8.5cm): d = 6.50cm → f_alias = 2,638 Hz (primary ring — PETG blue)

- RING-C (15cm): d = 11.48cm → f_alias = 1,494 Hz (PETG green)

Color coding eliminates wrong-ring installation; visually obvious from across the room.

## 6.5 Complete Bill of Materials

| ID | Item | Spec | Qty | Unit | Total | Justification |
| --- | --- | --- | --- | --- | --- | --- |
| B-001 | Behringer ECM8000 | Measurement mic, omni | 8 | $29 | $232 | Selected in Sec 6.1 — lowest cost measurement mic meeting FR-001–FR-004 |
| B-002 | XLR cable 3ft | Balanced, identical length | 8 | $3 | $24 | Identical lengths required for consistent electrical characteristics |
| B-003 | Bambu Lab P1S | FDM printer, enclosed | 1 | $399 | $399 | PETG capability required for moisture/temp resistant mounts |
| B-004 | PETG filament | 1kg spool | 2 | $25 | $50 | Final production mounts — 2 spools for all 5 mounts + 3 rings |
| B-005 | PLA filament | 1kg spool | 1 | $25 | $25 | Prototyping and fit checks |
| B-006 | M3 thumbscrews | Collar clamp fasteners | 16 | $0.50 | $8 | Toolless mic collar release — FR-007 |
| B-007 | M3 heat-set inserts | Threaded inserts | 16 | $0.30 | $5 | Durable threads in 3D printed parts |
| B-008 | 5/8"-27 adapter | Mic stand thread adapter | 1 | $5 | $5 | Stand compatibility — FR-008 |
| B-009 | Label tape | Cable identification | 1 | $8 | $8 | Channel assignment consistency |
| B-010 | Misc fasteners | Assembly hardware | — | — | $10 | Contingency |
|  |  | TOTAL NEW EQUIPMENT |  |  | $766 |  |

Items already owned (no cost):

| Item | Estimated Value | Role in PING |
| --- | --- | --- |
| TASCAM US-16x08 | ~$250 | 8-channel audio interface — core DAQ system |
| Custom PC build | ~$2,400 | Primary computation and software development host |
| Microphone stand (5/8"-27) | ~$30 | Array mounting |
| Onshape CAD (free tier) | $0 | All mechanical design |
| Python 3.10+, GitHub, PyRoomAcoustics | $0 | Software development and version control |

# SECTION 7 — SOFTWARE DESIGN SPECIFICATION

## 7.1 Design Philosophy

Four principles govern every software decision in PING:

- Reproducibility above all else — every experiment must produce identical results when run again with the same data and configuration. Fixed seeds, no hardcoded parameters, complete logging.

- Separation of concerns — hardware, signal processing, algorithms, and visualization never share code. A change in one layer never requires changes in another.

- Explicit over implicit — every parameter is named, documented, and configurable. No magic numbers. If a value of 512 appears in the codebase, a comment explains why 512 and not 256 or 1024.

- Research-grade documentation — every function has a docstring. Every module has a header. Every algorithm has a corresponding theory document.

## 7.2 Development Environment

Language: Python 3.10+. Justification: Python is the dominant language in acoustic signal processing research. NumPy, SciPy, and PyRoomAcoustics are Python-native. Interactive Jupyter notebooks enable iterative algorithm development.

| Library | Version | Purpose |
| --- | --- | --- |
| numpy | ≥1.24.0 | Array operations, linear algebra |
| scipy | ≥1.10.0 | Signal processing, FFT, filtering |
| matplotlib | ≥3.7.0 | Visualization, publication figures |
| sounddevice | ≥0.4.6 | Real-time audio I/O |
| pyroomacoustics | ≥0.7.3 | DOA algorithms, room simulation |
| librosa | ≥0.10.0 | Audio analysis utilities |
| h5py | ≥3.8.0 | Structured data storage |
| pyyaml | ≥6.0.0 | Configuration file parsing |
| pytest | ≥7.3.0 | Automated unit and integration testing |

## 7.3 Package Structure

PING is a structured Python package, not a collection of disconnected scripts. Every folder represents a subsystem with a clear responsibility:

- oaap/hardware/ — all TASCAM communication; no other module calls sounddevice directly

- oaap/calibration/ — CLAP protocol and sensitivity gain management

- oaap/geometry/ — mic position definitions for all 5 geometries (shared base class)

- oaap/signal_processing/ — acquisition, DC removal, bandpass filter, windowing

- oaap/beamforming/ — GCC-PHAT, DAS, MVDR, MUSIC, SRP-PHAT (shared base class)

- oaap/visualization/ — polar heatmap, spectrum plots, comparison figures

- experiments/ — structured experiment scripts (EXP-001 through EXP-012)

- data/ — calibration gains, raw recordings (not committed to git)

- results/ — figures and experiment reports (committed to git)

- docs/ — technical report, theory documents, images

- tests/ — pytest unit and integration tests

## 7.4 Standardized Interfaces

Every geometry module and every beamforming algorithm implements the same base class interface. This means swapping geometry or algorithm is a single line change in any experiment script:

- ArrayGeometry base class: get_positions() → (2×N) array, get_name(), get_aliasing_frequency(), plot()

- BeamformingAlgorithm base class: locate_sources(signals) → (360,) spatial spectrum array, get_name()

With this design: algorithm = MUSICAlgorithm(...) swaps to algorithm = DelayAndSumAlgorithm(...) with one word changed. Everything else is identical.

## 7.5 Configuration System

All parameters live in ping_config.yaml — never hardcoded. Every experiment runs with its own config file, making experiments fully reproducible and parameterized:

- hardware: device name, sample rate, bit depth, channels, phantom power, buffer size

- array: geometry type, radius (cm), n_mics, speed of sound

- processing: window type, NFFT, overlap, bandpass frequencies

- algorithm: primary algorithm, n_sources, angular resolution

- visualization: refresh rate, colormap, spectrum display toggle

- clap: duration, threshold, max failed channels

- logging: save raw audio, save figures, output directory

## 7.6 CLAP Protocol Logic

CLAP (Channel Level Acquisition Protocol) executes at every PING startup:

- Step 1 ANNOUNCE: print startup message, prompt user to clap once

- Step 2 LISTEN: record 3 seconds of audio, all 8 channels

- Step 3 DETECT: find peak amplitude per channel, compare to –40 dBFS threshold

- Step 4 REPORT: print channel-by-channel PASS/FAIL table with peak and RMS levels

- Step 5 EVALUATE: ALL PASS → continue; 1–2 FAIL → warn user, prompt to continue; 3+ FAIL → system fault, exit

- Step 6 CALIBRATE: compute sensitivity correction gains from CLAP RMS levels, save to disk

## 7.7 Testing Strategy

All software verification is automated via pytest. The test suite runs before every experiment session to ensure no regressions. Key tests include:

- test_circular_positions_shape: verifies positions array is (2, 8)

- test_circular_positions_on_circle: verifies all mics lie at specified radius

- test_circular_equal_spacing: verifies 45° equal angular spacing

- test_random_reproducibility: verifies seed=42 produces identical positions across sessions (FR-020)

- test_known_tdoa_estimation: creates synthetic signal delayed 10 samples, verifies GCC-PHAT estimates 10 samples (FR-014)

- test_full_pipeline_runs: end-to-end integration test from synthetic audio to spatial spectrum

## 7.8 Code Style Standards

Every function follows a consistent docstring format: one-sentence description, longer explanation of WHY not just WHAT, Args with types and units, Returns with shape for arrays, Raises, Notes. No magic numbers; every constant is named and commented with justification.

# SECTION 8 — EXPERIMENTAL PLAN

## 8.0 Experimental Framework

Every PING experiment follows a standardized structure:

- Experiment ID, Title, Category, Status, Dependencies

- Question: the single scientific question this experiment answers

- Hypothesis: the expected answer with physical justification

- Requirements: which functional requirements this experiment verifies

- Setup: physical configuration — geometry, ring, distances, angles

- Stimulus: what acoustic source is used and how

- Procedure: step-by-step numbered instructions

- Data collected: exactly what is recorded and saved

- Analysis: how data is processed and what metrics are computed

- Success criteria: quantitative threshold for experiment to pass

- Figures: what plots are generated

- Report: location of written report

## 8.1 Experiment Summary

| ID | Title | Category | Depends On | Key Question |
| --- | --- | --- | --- | --- |
| EXP-001 | Multichannel Synchronization | Foundation | None | Are all 8 channels sample-accurate synchronized? |
| EXP-002 | CLAP Sensitivity Calibration | Foundation | EXP-001 | Does CLAP reduce sensitivity variation to ±1 dB? |
| EXP-003 | Baseline DOA Accuracy | Characterization | EXP-001, 002 | What is MUSIC accuracy vs angle and frequency? |
| EXP-004 | Aperture Ring Comparison | Characterization | EXP-003 | How does radius affect accuracy and frequency range? |
| EXP-005 | Algorithm Comparison | Comparison | EXP-003 | How do GCC-PHAT, DAS, MVDR, MUSIC, SRP-PHAT compare? |
| EXP-006 | Geometry Comparison | Comparison (Crown Jewel) | EXP-003,004,005 | Which geometry performs best for localization? |
| EXP-007 | Noise Suppression | Characterization | EXP-005 | Can MVDR achieve ≥6 dB SNR improvement? |
| EXP-008 | Car Fault Localization | Application | EXP-006 | Can PING localize an engine clunk? |
| EXP-009 | Acoustic Classification | Secondary (v2.0) | EXP-006 | Can PING classify sound events with ≥70% accuracy? |
| EXP-010 | Multi-Source Localization | Secondary (v2.0) | EXP-006 | Can PING locate two simultaneous sources? |
| EXP-011 | Acoustic Camera | Secondary (v2.0) | EXP-006 | Can PING overlay heatmap on live video? |
| EXP-012 | UAV Motor Fault Localization | Secondary (v2.0) | EXP-009,010 | Can PING identify which motor is faulting externally? |

## 8.2 Experiment Dependency Chain

The dependency chain enforces that foundation experiments are complete before characterization or comparison begins. No exceptions:

- EXP-001 (Sync) → EXP-002 (Calibration) → EXP-003 (Baseline DOA)

- EXP-003 → EXP-004 (Ring Comparison)

- EXP-003 → EXP-005 (Algorithm Comparison) → EXP-007 (Noise Suppression)

- EXP-003 + EXP-004 + EXP-005 → EXP-006 (Geometry Comparison — Crown Jewel)

- EXP-006 → EXP-008 (Car Application)

- EXP-006 → EXP-009, 010, 011, 012 (v2.0 secondary experiments)

## 8.3 EXP-006 Detail — The Crown Jewel Experiment

EXP-006 (Array Geometry Comparison) is the most significant experiment in PING because it produces a side-by-side comparison of all five array geometries under identical conditions. This is the experiment that answers definitively which geometry is best for which application, a genuinely open research question.

- Fixed variable: MUSIC algorithm (same for all geometries)

- Fixed variable: RING-B equivalent aperture for all geometries

- Fixed variable: source at 45°, 1.0 meter distance

- Variable: only the microphone positions change

- Test frequencies: 500 Hz, 1000 Hz, 2000 Hz

- Trials: 10 per (geometry × frequency) = 150 total recordings

Key figures: side-by-side heatmaps of all 5 geometries on identical input; the most compelling single figure in PING and the one most likely to attract attention from NASA, defense contractors, and audio engineering employers.

## 8.4 EXP-008 — Car Acoustic Fault Localization

This application experiment serves dual purpose: (1) validates PING in a real-world field scenario; (2) potentially diagnoses an actual vehicle fault. The linear array (MOUNT-2) is positioned across the undercarriage of the author’s 2013 Chevrolet Camaro RS, which developed a clunk after an auto shop visit. Results are documented regardless of outcome, negative results are part of the scientific record.

# SECTION 9 — VALIDATION AND VERIFICATION

## 9.1 Verification vs Validation

- Verification: Did we build the system right? — Does the software do what the specification says? Addressed through unit tests, hardware checks, and experiment results.

- Validation: Did we build the right system? — Does the system actually solve the problem it was designed to solve? Addressed through EXP-012 (UAV Motor Fault Localization).

## 9.2 Master Verification Matrix

| FR-ID | Requirement Summary | Verification Method | Experiment | Status |
| --- | --- | --- | --- | --- |
| FR-001 | 8-channel sync | Timing offset measurement | EXP-001 | ⏳ |
| FR-002 | 44,100 Hz sample rate | Device query | EXP-001 | ⏳ |
| FR-003 | 24-bit depth | Dynamic range test | EXP-001 | ⏳ |
| FR-004 | Sensitivity ±1 dB | Pre/post RMS | EXP-002 | ⏳ |
| FR-005 | 48V phantom power | Multimeter measurement | Hardware | ⏳ |
| FR-006 | Geometry swap < 10min | Timed swap | Physical | ⏳ |
| FR-007 | Ring swap < 2min | Timed swap | Physical | ⏳ |
| FR-008 | Stand compatibility | Physical fit test | Physical | ⏳ |
| FR-009 | Cable clearance | Ruler measurement | Physical | ⏳ |
| FR-010 | Collar on all mounts | Physical fit test | Physical | ⏳ |
| FR-011 | Latency < 500ms | Impulse to display | EXP-001 | ⏳ |
| FR-012 | Geometry (2×N) array | Unit test | pytest | ⏳ |
| FR-013 | Calibration persists | Restart and verify | EXP-002 | ⏳ |
| FR-014 | GCC-PHAT < 2 samples | Known TDOA signal | EXP-003 | ⏳ |
| FR-015 | DAS < ±5° | Known angle | EXP-003 | ⏳ |
| FR-016 | MUSIC < ±5° | Known angle | EXP-003 | ⏳ |
| FR-017 | Heatmap ≤1°, ≥2Hz | Code + visual | EXP-003 | ⏳ |
| FR-018 | Data logging | File inspection | All EXPs | ⏳ |
| FR-019 | CLAP < 10s | Timed execution | EXP-002 | ⏳ |
| FR-020 | Random reproducible | Hash comparison | pytest | ⏳ |
| FR-021 | 7 report sections | Document review | All EXPs | ⏳ |
| FR-022 | Figures ≥150 DPI | File metadata | All EXPs | ⏳ |
| FR-023 | CAD STEP files | Repository check | GitHub | ⏳ |
| FR-024 | Theory docs | Document review | docs/ | ⏳ |
| FR-025 | README cold start | Independent test | GitHub | ⏳ |
| FR-026 | Technical report | Document review | Final | ⏳ |

Status legend: ⏳ Planned  🔄 In Progress  ✅ PASS  ⚠️ WARN  ❌ FAIL

## 9.3 Hardware Verification Procedures

Five physical verification procedures are documented:

- HV-001 — Phantom Power Verification (FR-005): Measure voltage on all 8 XLR channels with multimeter. Pass: 44–52V on all channels simultaneously.

- HV-002 — Geometry Swap Time (FR-006): Timed swap from MOUNT-1 to MOUNT-2 and back, 3 repetitions. Pass: < 10 minutes.

- HV-003 — Ring Swap Time (FR-007): Timed swap of RING-B to RING-A without tools. Pass: < 2 minutes.

- HV-004 — Stand Compatibility (FR-008): Thread MOUNT-1 onto 5/8"-27 stand, apply lateral force. Pass: no play or wobble.

- HV-005 — Cable Clearance (FR-009): Ruler measurement of cable-to-capsule distance at all 8 mics. Pass: > 5cm on all mics.

## 9.4 Validation Approach

PING is fully validated when EXP-012 demonstrates that an external array can localize acoustic fault signatures to a specific motor on a multi-rotor UAV frame without any instrumentation attached to the motors themselves. Until EXP-012 is complete, PING is verified but not fully validated. This distinction is documented explicitly as a matter of intellectual honesty.

## 9.5 Known Limitations (v1.0)

| ID | Limitation | Impact | Future Mitigation |
| --- | --- | --- | --- |
| L-001 | Indoor operation only | Wind and outdoor reflections degrade performance | Weatherproofing in v2.0 |
| L-002 | Single source assumption (basic algorithms) | Performance degrades with multiple equal-level sources | MUSIC handles multiple sources with n_src parameter |
| L-003 | Far-field assumption (> 0.5m for RING-B) | Near-field sources produce systematic localization error | Near-field correction in v2.0 |
| L-004 | Frequency limited by spatial aliasing | Accurate only below ring-specific aliasing frequency | Smaller radius ring or higher sample rate |
| L-005 | Static array (no source tracking) | Cannot track moving sources | Source tracking in v2.0 |
| L-006 | Reverberation sensitivity | Accuracy degrades in highly reverberant environments | SRP-PHAT most robust; dereverberation in v2.0 |

## 9.6 Version Release Criteria

| Version | Milestone | Criteria |
| --- | --- | --- |
| v0.1 | Design Complete | PRDS document complete, GitHub initialized, hardware ordered |
| v0.5 | Hardware Validated | HV-001–HV-005 PASS, EXP-001 PASS, EXP-002 PASS, real-time heatmap displaying |
| v1.0 | Primary Objectives Complete | All FR SHALL requirements verified PASS, EXP-001–EXP-008 complete, technical report draft, GitHub public |
| v2.0 | Secondary Objectives | EXP-009–EXP-011 complete, classification system, acoustic camera polished |
| v3.0 | Full Vision | EXP-012 UAV motor fault localization, distributed network operational, open source release |

# SECTION 10 — PROJECT SCHEDULE

## 10.1 Schedule Philosophy

PING follows an iterative milestone approach rather than a strict Gantt chart. Each milestone has a target date and a definition of done. Missing a target date means updating the schedule honestly and continuing, not abandoning the project. One non-negotiable rule: foundation experiments (EXP-001, EXP-002) must be complete before any characterization or comparison experiments begin.

## 10.2 Schedule Constraints

| Constraint | Impact |
| --- | --- |
| Hardware delivery (ECM8000s, P1S printer) | ~1–2 weeks from order |
| 3D print time for all mounts and rings | ~2–3 weeks total |
| Overnight work schedule (Sam’s Club) | Limits daytime build/experiment time |
| NASA Langley internship (Aug 24 – Dec 4, pending) | 15 weeks of reduced PING availability |
| Graduate school prep | Parallel track, minimal PING impact |
| Current date | July 2026 |

## 10.3 Phase Summary

| Phase | Period | Duration | Focus |
| --- | --- | --- | --- |
| Phase 0 | July 2026 | 2 weeks | Planning, procurement, GitHub setup |
| Phase 1 | July 25 – Aug 20, 2026 | 4 weeks | Hardware build, CAD, software setup |
| Phase 2 | Aug 20–24, 2026 | 4 days | Foundation experiments EXP-001 & EXP-002 |
| Phase 3 | Aug 24 – Dec 4, 2026 | 15 weeks | NASA Langley internship (reduced PING pace) |
| Phase 4 | Dec 2026 – Jan 2027 | 6 weeks | Characterization experiments EXP-003–EXP-005 |
| Phase 5 | Jan – Feb 2027 | 5 weeks | Comparison experiments EXP-006–EXP-007 |
| Phase 6 | Feb – Mar 2027 | 4 weeks | Application experiments EXP-008, acoustic camera |
| Phase 7 | Mar – Apr 2027 | 4 weeks | Documentation, technical report, public release |
| Phase 8 | Apr 2027+ | Ongoing | PING v2.0 planning and execution |

## 10.4 Key Milestones

| Milestone | Target Date | Definition of Done |
| --- | --- | --- |
| PRDS v0.1 complete | July 25, 2026 | All 12 sections written, committed to GitHub |
| GitHub repository live | July 20, 2026 | Folder structure pushed, README skeleton public |
| Hardware received | August 1, 2026 | ECM8000s and P1S printer inspected, no damage |
| PING v0.5 achieved | August 24, 2026 | EXP-001 & EXP-002 PASS, real-time heatmap operational |
| Algorithm suite complete | December 20, 2026 | GCC-PHAT, DAS, MVDR, MUSIC, SRP-PHAT — all pytest PASS |
| Crown jewel experiment | February 10, 2027 | EXP-006 geometry comparison complete, all figures generated |
| PING v1.0 released | April 10, 2027 | Public GitHub release, technical report complete, README cold-start verified |
| LinkedIn announcement | April 11, 2027 | Public post with heatmap demo GIF, resume updated |

## 10.5 NASA Langley Internship Period

During the internship, if offered, (August 24 – December 4, 2026), PING development continues at reduced pace. The internship is in acoustic measurement, microphone proximity effects in direct field acoustic tests, and insights from Langley will flow back into PING’s documentation and experimental design. PING v0.5 is operational before the internship begins, meaning a live GitHub repository with working code backs up any conversation about PING at Langley.

## 10.6 Progress Tracking

- GitHub Projects board — columns: Backlog / In Progress / Review / Complete. Every schedule task becomes a GitHub Issue linked to milestones and version tags.

- PING Experiment Log — updated after every work session, committed to GitHub regardless of productivity. Failed experiments documented, not hidden.

- Weekly Sunday check-in — review schedule, update GitHub board, commit pending documentation. Keeps repository active and shows consistent engagement.

# SECTION 11 — BUDGET

## 11.1 Budget Philosophy

PING is designed to be a research-grade platform at hobbyist-accessible cost. Every hardware decision in Section 6 explicitly considered cost as a design constraint alongside technical performance. The goal: demonstrate that meaningful acoustic array research can be conducted for under $1,000 in new equipment — making the platform reproducible by other researchers without institutional funding.

## 11.2 New Equipment Budget

| ID | Item | Qty | Unit Cost | Total | Justification |
| --- | --- | --- | --- | --- | --- |
| B-001 | Behringer ECM8000 | 8 | $29 | $232 | Lowest cost measurement mic meeting all requirements |
| B-002 | XLR cable 3ft (balanced) | 8 | $3 | $24 | Identical lengths — equal electrical characteristics |
| B-003 | Bambu Lab P1S (enclosed) | 1 | $399 | $399 | PETG capability for moisture/temperature resistant mounts |
| B-004 | PETG filament 1kg | 2 | $25 | $50 | Final production mounts — all 5 mounts + 3 rings |
| B-005 | PLA filament 1kg | 1 | $25 | $25 | Prototyping and fit checks before PETG |
| B-006 | M3 thumbscrews | 16 | $0.50 | $8 | Toolless mic collar release |
| B-007 | M3 heat-set inserts | 16 | $0.30 | $5 | Durable threads in printed parts |
| B-008 | 5/8"-27 adapter | 1 | $5 | $5 | Mic stand compatibility |
| B-009 | Label tape | 1 | $8 | $8 | Channel assignment consistency |
| B-010 | Misc fasteners | — | — | $10 | Assembly contingency |
|  | TOTAL |  |  | $766 |  |

## 11.3 Existing Equipment (Zero Additional Cost)

| Item | Estimated Value | Role in PING |
| --- | --- | --- |
| TASCAM US-16x08 | ~$250 | 8-channel DAQ interface — core acquisition system |
| Custom PC build | ~$2,400 | Primary computation and software development |
| Microphone stand (5/8"-27) | ~$30 | Array mounting |
| Onshape, Python, GitHub | $0 | CAD, software, version control |
| Total existing equipment value | ~$2,680 |  |

## 11.4 Cost vs Commercial Alternatives

| System | Capability | Cost |
| --- | --- | --- |
| Brüel & Kjær PULSE Array | Research-grade industrial measurement | $50,000 – $200,000+ |
| mh acoustics em32 Eigenmike | 32-channel spherical array | ~$8,000 |
| GRAS Sound & Vibration Array | Measurement array systems | $15,000 – $80,000+ |
| PING v1.0 | 8-channel modular research platform | $766 new / ~$3,446 total |

PING does not claim equivalent performance to $200,000 research systems. What PING demonstrates is that the fundamental acoustic array research concepts; beamforming, DOA estimation, geometry comparison; can be meaningfully investigated at a fraction of commercial system cost. This democratization of acoustic array research is itself a contribution worth stating explicitly.

## 11.5 Return on Investment

For PING as a portfolio project, return is career-oriented rather than financial:

- Demonstrated independent research capability — unquantifiable career asset

- Public GitHub repository — permanent, visible to any employer worldwide

- Conference paper potential — academic credibility and peer validation

- Interview talking point — distinguishes from every other candidate at the same experience level

Comparable investments: graduate school application fees ($50–200 each), online DSP courses ($200–2,000), professional conference attendance ($500–2,000). PING at $766 produces a living, growing, publicly visible research platform that directly supports the career objectives described in Section 2; no comparable investment of similar magnitude exists.

## 11.6 Future Budget (PING v2.0)

| Item | Purpose | Estimated Cost |
| --- | --- | --- |
| Raspberry Pi 4 ×3 | Edge computing for distributed network | ~$225 |
| GPS modules ×3 | Time synchronization for DASN | ~$90 |
| Weatherproof enclosures ×3 | Outdoor DASN deployment | ~$120 |
| Wireless networking | Inter-unit communication | ~$50 |
| Additional ECM8000 sets | Second and third PING units | ~$256 each |
| Total estimated v2.0 |  | ~$535–$791 |

# SECTION 12 — REFERENCES AND RELATED WORK

## 12.1 Foundational References

### Array Signal Processing — Foundational Texts

- [1] Van Trees, H.L. (2002). Optimum Array Processing: Part IV of Detection, Estimation, and Modulation Theory. John Wiley & Sons, New York. — The definitive reference for array signal processing theory. Covers delay-and-sum beamforming, MVDR, and spatial spectrum estimation.

- [2] Johnson, D.H. and Dudgeon, D.E. (1993). Array Signal Processing: Concepts and Techniques. Prentice Hall. — Accessible treatment of spatial aliasing, array gain, and beampattern design. The spatial aliasing derivation in Section 6.4 follows Chapter 2.

- [3] Brandstein, M. and Ward, D. (Eds.) (2001). Microphone Arrays: Signal Processing Techniques and Applications. Springer, Berlin. — Practical guide to hardware, calibration, and real-world deployment. Directly relevant to the CLAP protocol design.

### Direction of Arrival Estimation

- [4] Schmidt, R.O. (1986). Multiple Emitter Location and Signal Parameter Estimation. IEEE Transactions on Antennas and Propagation, 34(3), 276–280. — The original MUSIC algorithm paper. PING’s MUSIC implementation is based directly on the eigendecomposition approach described here.

- [5] Capon, J. (1969). High-Resolution Frequency-Wavenumber Spectrum Analysis. Proceedings of the IEEE, 57(8), 1408–1418. — The original MVDR (Capon beamformer) paper. PING’s MVDR implementation traces directly to this work.

- [6] Knapp, C.H. and Carter, G.C. (1976). The Generalized Correlation Method for Estimation of Time Delay. IEEE TASSP, 24(4), 320–327. — The original GCC paper introducing PHAT weighting. Foundation for PING’s first implemented algorithm.

- [7] DiBiase, J.H., Silverman, H.F., and Brandstein, M.S. (2001). Robust Localization in Reverberant Rooms. In: Brandstein & Ward (Eds.), Microphone Arrays, Springer, 157–180. — Introduces SRP-PHAT. PING’s SRP-PHAT follows the grid-based spatial scanning approach described here.

### Acoustic Scene Classification and Machine Learning

- [8] LeCun, Y., Bengio, Y., and Hinton, G. (2015). Deep Learning. Nature, 521, 436–444. — Foundational deep learning reference. PING’s planned classification system builds on CNN principles developed here and in the author’s NASA Ames work.

- [9] Salamon, J., Jacoby, C., and Bello, J.P. (2014). A Dataset and Taxonomy for Urban Sound Classification. ACM Multimedia, 1041–1044. — Introduces systematic acoustic scene classification methodology relevant to PING’s OBJ-S7 design.

- [10] Mesaros, A. et al. (2017). DCASE 2017 Challenge Setup: Tasks, Datasets, and Baseline System. DCASE Workshop. — The benchmark for acoustic classification research. PING’s classification system will be evaluated against DCASE methodology.

### UAV Acoustics and Health Monitoring

- [11] Krishnamurthy, A. et al. (2021). Acoustic Detection and Tracking of UAVs. JASA, 150(4), A236. — Demonstrates acoustic-based UAV detection using microphone arrays — the approach PING applies to motor fault localization.

- [12] Yoon, S. et al. (2020). Fault Detection of Brushless DC Motors Using Acoustic Signal Analysis. IEEE TIE, 67(11), 9499–9507. — Acoustic fault detection for brushless DC motors — the exact application domain of the author’s NASA Ames internship.

- [13] Shi, W. et al. (2022). Physics-Informed Neural Networks for Motor Fault Detection. Mechanical Systems and Signal Processing, 174, 109063. — Directly relevant to the PINN modeling approach from the author’s NASA Ames internship.

### Distributed Acoustic Sensor Networks

- [14] Cobos, M. et al. (2017). A Survey of Sound Source Localization with Deep Learning Methods. JAES, 65(7/8), 559–571. — Comprehensive survey including distributed sensor network approaches. Relevant to PING’s OBJ-S8 design.

- [15] Knudsen, M.V. et al. (2021). Gunshot Detection Using Distributed Microphone Arrays. Applied Acoustics, 172, 107614. — Demonstrates triangulation accuracy with distributed acoustic sensors — primary reference for PING’s distributed network design.

### Software and Tools

- [16] Scheibler, R., Bezzam, E., and Dokmanić, I. (2018). Pyroomacoustics: A Python Package for Audio Room Simulation and Array Processing. ICASSP 2018, 351–355. — The PyRoomAcoustics library used throughout PING’s signal processing stack.

- [17] Harris, C.R. et al. (2020). Array Programming with NumPy. Nature, 585, 357–362. — Required citation when NumPy is used in published or publicly released research code.

- [18] Virtanen, P. et al. (2020). SciPy 1.0: Fundamental Algorithms for Scientific Computing in Python. Nature Methods, 17, 261–272. — Required citation for SciPy use in published research.

- [19] Hunter, J.D. (2007). Matplotlib: A 2D Graphics Environment. Computing in Science & Engineering, 9(3), 90–95. — Required citation for Matplotlib figures in published work.

## 12.2 Related Work

### Commercial Systems

- Brüel & Kjær PULSE Array Systems ($50,000–$200,000+): The B&K PULSE system is the gold standard for industrial acoustic array measurement. PING makes no claim to comparable accuracy or industrial certification. What PING offers that B&K does not: open-source algorithms, modifiable geometry, transparent signal processing, and full reproducibility.

- mh acoustics em32 Eigenmike (~$8,000): 32-channel spherical array for 3D spatial audio and HOA research. PING operates in 2D with 8 channels — less capable spatially but dramatically more accessible and modifiable. The Eigenmike is a finished product; PING is a research platform where every component is understood and changeable.

- ShotSpotter (Commercial gunshot detection): Uses 15–25 sensors per square mile, achieves ~25-meter localization accuracy. PING’s OBJ-S8 is a three-unit research prototype — not a competitive product but an investigation of the same fundamental distributed sensor principles.

### Open Source and Research Platforms

- Microsoft Acoustic Platform (GitHub): Microsoft Research has released acoustic array tools including beamforming implementations. PING’s software architecture was informed by reviewing Microsoft’s approach to module separation and interface design; PING’s implementation is entirely original.

- ReSpeaker (Seeed Studio): Low-cost ($50–150) circular array for voice interface applications with DAS beamforming in firmware. PING differs in scope — ReSpeaker is optimized for speech pickup in consumer devices; PING is optimized for research flexibility and algorithm comparison across multiple geometries.

### PING’s Unique Contribution

After reviewing commercial and open research platforms, PING occupies a specific niche that no existing system addresses identically:

- Modular geometry system — five interchangeable array geometries on a single microphone set, enabling controlled comparison without purchasing separate arrays. No commercial or open-source system reviewed offers this.

- Interchangeable aperture rings — three aperture sizes on the same circular geometry, enabling systematic study of the radius/frequency tradeoff. A novel hardware approach to a well-studied theoretical question.

- Research-grade documentation at hobbyist cost — PRDS-level documentation, structured experiments with hypotheses and success criteria, automated testing, and full open-source release under $1,000 in new equipment.

- NASA-motivated application — explicitly designed to investigate the UAV motor fault localization problem identified during the author’s NASA Ames internship. Real-world aerospace motivation distinguishes PING from general-purpose educational projects.

- CLAP protocol — a named, documented startup verification procedure integrating impulse testing into the initialization sequence. Novel in its combination of practical hardware verification and acoustic characterization.

## 12.3 Author’s Prior Work

- [A1] Jackson, J. (2025). Acoustic Fault Detection for Uncrewed Aerial Platforms Using Physics-Informed Neural Networks. Presented at NASA Ames Research Center Intern Poster Session, Moffett Field, CA. — Documents the acoustic health monitoring research that directly motivated PING. The multi-microphone testbed, FFT-based spectral analysis pipeline, and CNN fault classification system described in that work are the foundational context for PING’s development. PING is the next research step beyond [A1] — moving from per-motor instrumentation with independent channels to a unified external array with spatial localization capability.

END OF DOCUMENT

PING — Portable Intelligent Noise Grid  |  PRDS v0.1  |  July 2026

Joshua Jackson  |  jsj830@outlook.com  |  linkedin.com/in/joshua-jackson-40a46225a